1

	
[image: image1.png]



	CETOS, a.s., Brojova 16 / 2113  PLZEN, 326 00

Kontaktni adresa/ Contact Address:
CETOS, a.s., U Tovaren 31, 102 12 Praha 10 - Hostivar 

Tel.: +420 296 583 527, 625  Fax: +420 296 583 437

E-mail: sales@cetos.cz , dsrimpl@cetos.cz , jsvobodova@cetos.cz , vchvatlina@cetos.cz  

Internet: www.cetos.cz , www.toscelakovice.cz 
	
[image: image2.wmf]

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	Naše značka
	Vyřizuje / linka
	V Čelákovicích, dne

	P2125/05
	Stebra
	19.8.2005


Offer No. P2125/05 – universal centre lathe for fine mechanics MN 80 A

Dear sir,

We are sending attached the pricelist of universal centre lathe MN 80 A. 

 

FINE MECHANICS LATHE    

MN 80 A
The MN80 A centre lathe is designed for machining components of any metal or palstic. It features a wide range of spindle speeds, facility of thread cuttings, its accessories extending its working range also to non-turning operations.The lathe is mounted on a wooden cabinet-type bench. The left hand bench section housed the drive motor, the switch to start, stop and reverse the motor run and the fuses. The remaining space of the bench is reserved for keeping operating tools and accessories.

The main spindle is mounted on adjustable plain bearings. Nine spindle speed steps are obtained through ”V” belts, three-step pulleys and countershaft engaging.

The carriage consists of the longitudinal slide, cross slide and swivel tool slide. The bed ways re hardened and precision-ground. Clearance between the slides and their guideways is eliminated by means of taper gibs. The slide feed screws are provided with indexing collars. The feed screw nuts are adjustable to eliminate backlash. The four –way tool post is indexable.

Capacities of the machine

Swing over bed/cross slide                                    
mm 
                                      160/90

Distance between centres                                       
mm 
                                            320

Width of bed                                                            
mm 
                                            120

Output of main drive motor                                      
kW 
                                            0,55

Weight of machine complete                                     
kg 
                                               180

Main Spindle

Spindle nose                                                                                         
                    M 39 x 4

Hole through spindle                                          
dia. mm                                         
          18

Taper in spindle                                                  
metric          
                                            25

Spindle speeds: 9 steps in the range of               
rpm 
                                            150 – 2000

Feeds and threads

19 longitudinal feed rates within                       
mm/rev                     
                   0,01 – 0,15

19 metric threads range of pitches                           
mm                       
                        0,2 – 3

15 Whitworth threads                                             
t.p.i.                      
                           36 – 7

17 module threads                                               
module                 
                           0,2 – 1,5

18 Diametral Pitch threads                                      
D.P.                      
                         20 – 80

Tailstock 

Taper in tailstock barrel                                                                                                   
Morse 1  

Travel of tailstock barrel                                         
mm                                                   
      80

Diameter of tailstock barrel                                     
mm                                                 
       20 

	CETOS a.s.   
	MN 80 A

	
	 EUR 

	
	

	 Price of basic machine MN 80 A including standard equipment 
	4 666

	
	

	 Standard equipment 
	

	 Electric motor with countershaft and motor run reverse switch 
	

	 Wired for 3x 400V/ 50Hz
	

	 Collet chuck attachment operable while the machine is running incl. 1 collet 
	

	 Chip pan 
	

	 2 lathe centres (MN 801) 
	

	 Backplate for scroll chuck 
	

	 Set of change gears to cut metric threads 
	

	 Machine operating tools 
	

	 Wooden cabinet - type bench 
	

	 Reducing socket for lathe centres (MN 807) 
	

	 Driving plate (MN 812) 
	

	 Operating instruction (2 copies) 
	

	
	

	 Option: 
	

	 MN 801 Normal centre MC1
	13

	 MN 802 Half centre MC1 
	13

	 MN 803 Hollow centre MC1
	26

	 MN 804 Hollow half - centre MC1
	26

	 MN 805 Driver-type centre for wood working MC1
	26

	 MN 806 Boring plate for round type component 
	13

	 MN 807 Centre sleeve metric-MC1
	26

	 MN 808 3-jaw chuck - 6 mm dia 
	26

	 MN 809 Set of collets dia 1 to 10mm 
	442

	 MN 810 Set of five stepped collets 10-40mm dia 
	273

	 MN 811 Set of five ring type collets 5 pcs 6-50mm dia 
	260

	 MN 812 Driving plate 
	26

	 MN 813 Polishing plate 
	26

	 MN 814 Flat drilling plate 
	26

	 MN 815 Face plate with 4 jaws dia 150 mm 
	143

	 MN 816 3-jaw chuck dia 80 mm 
	91

	 MN 817 4-jaw chuck dia 80 mm 
	104

	 MN 818 Lever - controlled drilling tailstock 
	247

	 MN 819 Swinging - away work rest 
	78

	 MN 820 Indexing attachment for work spindle 
	91

	 MN 822 Steady rest 
	104

	 MN 823 Travelling steady 
	91

	 MN 828 Height-adjustable carriage without angle plate 
	195

	 MN 829 Clamping angle plate 
	39

	 MN 830 Dividing attachment with flange 
	221

	 MN 831 Flange of 84 mm dia 
	26

	 MN 832 Gears for cutting inch. dp and module thread pit. 
	182

	 MN 834 Live centre MC1 
	34

	 MN 835 Guard of scroll chuck, driving plate and faceplate Fitting of jaw chuck flange 
	91

	 2 spare bronze bearing shells 
	130

	 set of easily wearable parts 
	169

	 10389 Bush 
	26

	 421113 Reducing socket 
	39

	 39182 Nut 
	39

	 Fitting of jaw chuck flange 
	13

	 Complete price for optional extras included of fitting of jaw chuck flange 
	3380

	 Packing – wooden box
	273

	 Packing – wooden crate
	65

	
	

	 Prices without VAT
	

	 FCA CETOS a.s. 
	


Delivery time:  3 months from the bind order
Payment:  100% before shipment of the machine, with possibility to provide inspection of the machine in our factory 
Best regards,

Milan Brincil

Sales Department






CETOS, a. s. byl zapsán do OR vedeného Krajským soudem v Plzni, pod spisovou značkou oddíl B, vložka 1110
DIČ/ VAT No./ Ust.ID.-Nr.  CZ61057711
2 

_1150103847.doc
[image: image1.png]






_1044779952.doc



